CPE 403	ADV EMB SYS DES	 FALL 2019

TITLE:  Midterm 1

AUTHOR: Steven Lee #5003887117

GOAL:

- Task 1:

  - Interface the given MPU6050 IMU using I2C protocol to TivaC.

  - Print all accelerometer and gyro values on to the serial terminal.

- Task 2:

  - Interface the given MPU6050 IMU using I2C protocol to TivaC.

  - Plot all accelerometer and gyro values on to a Graph

- Task 3:

  - Implement a complementary filter to filter the raw accelerometer and gyro values.

  - Print all raw and filtered accelerometer and gyro values on to the serial terminal.

  - Implement the filter using IQMath Library.

- Task 4:

  - Implement a complementary filter to filter the raw accelerometer and gyro values.

  - Plot all raw and filtered accelerometer and gyro values on to a Graph

DELIVERABLES:

For the midterm, I interfaced the MPU6050 with I2C to communicate with TM4C123GH6PM. To display the gyroscope and accelerometer values, they were sampled and printed through UART.

COMPONENTS:

- I2C

  - Only two bus lines required (Serial data line SDA and Serial clock line SCL)

  - No Strict baud rate requirements.

  - Master/Slave relationships exist between all components.

  - One data bit transferred each clock pulse. The data on SDA must remain stable during high period of clock pulse.

  - Both SDA and SCL remain high when bus is not busy. High to Low indicate start condition while Low to High indicates stop condition

- UART

  - Clock generator.

  - Input and output shift registers.

  - Transmit and receive control.

  - Read and write logic.

- IQMath Library

  - Simplified porting from the floating-point simulations to a fixed point MCU

  - Math library for fixed-point processors speeds computation of floating-point values

  - Conversions from floating to fixed point are completed at the compile stage, so CPU cycles are not used.

- MPU6050

  - 3-axis Accelerometer and Gyroscope

  - Uses I2C protocol

SCHEMATICS:

IMPLEMENTATION:

- Initialize I2C

- Initialize UART

- Initialize MPU6050

- Interface MPU6050 and TM4C123GH6PM with I2C

- Read from MPU6050

- Filter readings from gyroscope and accelerometer

- Send serial data via UART

SCREENSHOTS:

Task 1:

Task 2:

Task 3:

Task 4:

CODE:

#include <stdbool.h>

#include <stdint.h>

#include <stdlib.h>

#include <stdio.h>

#include <stdarg.h>

#include <stdbool.h>

#include "sensorlib/i2cm_drv.h"

#include "sensorlib/hw_mpu6050.h"

#include "sensorlib/mpu6050.h"

#include "inc/hw_ints.h"

#include "inc/hw_memmap.h"

#include "inc/hw_sysctl.h"

#include "inc/hw_types.h"

#include "inc/hw_i2c.h"

#include "inc/hw_types.h"

#include "inc/hw_gpio.h"

#include "driverlib/gpio.h"

#include "driverlib/pin_map.h"

#include "driverlib/rom.h"

#include "driverlib/rom_map.h"

#include "driverlib/debug.h"

#include "driverlib/interrupt.h"

#include "driverlib/i2c.h"

#include "driverlib/sysctl.h"

#include "driverlib/uart.h"

#include "uartstdio.h"

#include "IQmathLib.h"

#include "math.h"

#define ACCELEROMETER_SENSITIVITY 8192.0

#define GYROSCOPE_SENSITIVITY 65.536

#define SAMPLE_RATE 0.01

#define dt 0.01

volatile bool g_bMPU6050Done;

tI2CMInstance g_sI2CMSimpleInst;

void ComplementaryFilter(short [3], short [3], float *, float *);

void ConfigureUART(void);

void MPU6050Callback(void *, uint_fast8_t);

void InitI2C0(void);

void I2CMSimpleIntHandler(void);

void DelayInMs(int);

int main(void) {

float pitch;

float roll;

float rawPitch;

float rawRoll;

float xA = 0;

float yA = 0;

float zA = 0;

float xG = 0;

float yG = 0;

float zG = 0;

float fAccel[3];

float fGyro[3];

short fAccelShort[3];

short fGyroShort[3];

tMPU6050 sMPU6050;

SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

InitI2C0();

ConfigureUART();

g_bMPU6050Done = false;

MPU6050Init(&sMPU6050, &g_sI2CMSimpleInst, 0x68, MPU6050Callback, &sMPU6050);

while (!g_bMPU6050Done);

g_bMPU6050Done = false;

MPU6050ReadModifyWrite(&sMPU6050,MPU6050_O_ACCEL_CONFIG,0xFF,MPU6050_ACCEL_CONFIG_AFS_SEL_4G,MPU6050Callback,&sMPU6050);

while (!g_bMPU6050Done);

g_bMPU6050Done = false;

MPU6050ReadModifyWrite(&sMPU6050,MPU6050_O_GYRO_CONFIG,0xFF,MPU6050_GYRO_CONFIG_FS_SEL_250,MPU6050Callback,&sMPU6050);

while (!g_bMPU6050Done);

g_bMPU6050Done = false;

MPU6050ReadModifyWrite(&sMPU6050,MPU6050_O_PWR_MGMT_1, 0x00,0x00,MPU6050Callback,&sMPU6050);

while (!g_bMPU6050Done);

g_bMPU6050Done = false;

MPU6050ReadModifyWrite(&sMPU6050,MPU6050_O_PWR_MGMT_2,0x00,0x00,MPU6050Callback,&sMPU6050);

while (!g_bMPU6050Done);

while (1)

{

g_bMPU6050Done = false;

MPU6050DataRead(&sMPU6050, MPU6050Callback, &sMPU6050);

while (!g_bMPU6050Done) {        }

MPU6050DataAccelGetFloat(&sMPU6050, &fAccel[0], &fAccel[1], &fAccel[2]);

MPU6050DataGyroGetFloat(&sMPU6050, &fGyro[0], &fGyro[1], &fGyro[2]);

//Set raw values to be printed

xA = fAccel[0];

yA = fAccel[1];

zA = fAccel[2];

xG = fGyro[0];

yG = fGyro[1];

zG = fGyro[2];

//S fAccel and fGyro to short

fAccelShort[0] = (short)fAccel[0];

fAccelShort[1] = (short)fAccel[1];

fAccelShort[2] = (short)fAccel[2];

fGyroShort[0] = (short)fGyro[0];

fGyroShort[1] = (short)fGyro[1];

fGyroShort[2] = (short)fGyro[2];

ComplementaryFilter(&fAccelShort[0], &fGyroShort[0], &pitch, &roll);

// Raw values

rawPitch = atan2f((float)fAccelShort[0], (float)fAccelShort[2]) * 180 / M_PI;

rawRoll = atan2f((float)fAccelShort[1], (float)fAccelShort[2]) * 180 / M_PI;

UARTprintf("Accel X: %d | Accel Y: %d | Accel Z: %d\n", (int)xA*1000, (int)yA*1000, (int)zA*1000);

UARTprintf("Gyro X: %d | Gyro Y: %d | Gyro Z: %d\n", (int)xG*1000, (int)yG*1000, (int)zG*1000);

UARTprintf("Raw Pitch: %d | Raw Roll: %d\n", (int)rawPitch, (int)rawRoll);

// Set filtered values to be printed

xA = (float)fAccelShort[0];

yA = (float)fAccelShort[1];

zA = (float)fAccelShort[2];

xG = (float)fGyroShort[0];

yG = (float)fGyroShort[1];

zG = (float)fGyroShort[2];

UARTprintf("Filtered Accel X: %d | Filtered Accel Y: %d | Filtered Accel Z:  %d\n", (int)xA*1000, (int)yA*1000, (int)zA*1000);

UARTprintf("Filtered Gyro X: %d | Filtered Gyro Y: %d | Filtered Gyro Z: %d\n", (int)xG*1000, (int)yG*1000, (int)zG*1000);

UARTprintf("Filtered Pitch: %d | Filtered Roll: %d\n", (int)pitch, (int)roll);

DelayInMs(1000);

}

}

void ComplementaryFilter(short accData[3], short gyrData[3], float *pitch, float *roll) {

float pitchAcc, rollAcc;

*pitch += ((float)gyrData[0] / GYROSCOPE_SENSITIVITY) * dt;

*roll += ((float)gyrData[1] / GYROSCOPE_SENSITIVITY) * dt;

int forceMagnitudeApprox = abs(accData[0]) + abs(accData[1]) + abs(accData[2]);

if (forceMagnitudeApprox > 8192 && forceMagnitudeApprox < 32768) {

pitchAcc = atan2f((float)accData[1], (float)accData[2]) * 180 / M_PI;

*pitch = *pitch * 0.98 + pitchAcc * 0.02;

rollAcc = atan2f((float)accData[0], (float)accData[2]) * 180 / M_PI;

*roll = *roll * 0.98 + rollAcc * 0.02;

}

}

void ConfigureUART(void) {

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

SysCtlPeripheralEnable(SYSCTL_PERIPH_UART0);

GPIOPinConfigure(GPIO_PA0_U0RX);

GPIOPinConfigure(GPIO_PA1_U0TX);

GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

UARTClockSourceSet(UART0_BASE, UART_CLOCK_PIOSC);

UARTStdioConfig(0, 115200, 16000000);

}

void MPU6050Callback(void *pvCallbackData, uint_fast8_t ui8Status) {

if (ui8Status != I2CM_STATUS_SUCCESS) {}

g_bMPU6050Done = true;

}

void InitI2C0(void) {

SysCtlPeripheralEnable(SYSCTL_PERIPH_I2C0);

SysCtlPeripheralReset(SYSCTL_PERIPH_I2C0);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOB);

GPIOPinConfigure(GPIO_PB2_I2C0SCL);

GPIOPinConfigure(GPIO_PB3_I2C0SDA);

GPIOPinTypeI2CSCL(GPIO_PORTB_BASE, GPIO_PIN_2);

GPIOPinTypeI2C(GPIO_PORTB_BASE, GPIO_PIN_3);

I2CMasterInitExpClk(I2C0_BASE, SysCtlClockGet(), true);

HWREG(I2C0_BASE + I2C_O_FIFOCTL) = 80008000;

I2CMInit(&g_sI2CMSimpleInst, I2C0_BASE, INT_I2C0, 0xff, 0xff, SysCtlClockGet());

}

void I2CMSimpleIntHandler(void) {

I2CMIntHandler(&g_sI2CMSimpleInst);

}

void DelayInMs(int ms) {

SysCtlDelay((SysCtlClockGet() / (3 * 1000)) * ms) ;

}